APOLLO SOLAR VENTURES INC.

NUTECH Hardware & Software Solutions

NUTECH School System

Application Documentation

cafeteria-pos.html

Staffed cashier checkout terminal

Prepared for

Bernard P. Jocson — Founder & President  ·  Developer: Carlos

Audience:  Developers & operators

Date:  22 June 2026

# Contents

# 1.  Overview

cafeteria-pos.html is the staffed canteen terminal. The cashier taps a student's card, sees the wallet balance, rings up menu items, charges the wallet, and prints a receipt. It also handles wallet top-ups, linking new cards, and reassigning a card to a new owner. It is the one surface confirmed working with the Arduino + PN532 RFID reader over USB.

# 2.  Screens & Tools

- Checkout: tap card to balance + menu, build cart, pay, receipt.

- Top-up: add money to a wallet.

- Enroll: link a blank card to an account.

- Reassign: move a card to a new owner (Admin PIN gated).

- Settings: webhook URL, RFID reader (HID / Arduino), demo flag, Admin PIN.

# 3.  Configuration

Defined at the top of the file (some also editable in Settings):

DEMO_MODE: false            // POS ships live
N8N_BASE: ".../webhook/cafeteria"
RFID_SOURCE: "hid"          // or "arduino"
ARDUINO_BAUD: 115200
REQUIRE_FP: false           // fingerprint removed (DigitalPersona offline)
ADMIN_PIN: "1234"           // gates the reassign tool

# 4.  Backend Actions

POSTs JSON { action, ... } to the shared /cafeteria webhook.

| Action | Sends / does |
| --- | --- |
| verify | card tap to identity + balance |
| purchase | charge wallet for the cart; writes sale + line items + decrements stock |
| topup | add money to a wallet |
| enroll | link a blank card to an account |
| menu | load menu items |
| account-info | look up a wallet (also serves check balance) |
| reassign | move a card to a new owner |

# 5.  Hardware

- RFID: Arduino + PN532 over USB (Web Serial, 115200). Confirmed working.

- Fingerprint: removed — checkout is card-only.

- Printer (optional): XP-58IIH via print-bridge on port 9002.

# Running & Serving

This is a single self-contained HTML file — no build step. The camera and the Arduino reader need a secure context, so serve it over http://localhost or HTTPS — never opened as a file.

## Option A — VS Code Live Server

- Install the Live Server extension.

- Open the apps/ folder, right-click this file, choose Open with Live Server.

- It opens at http://127.0.0.1:5500/… — a secure context, so camera + Web Serial work.

## Option B — Python

python -m http.server 8080
# then open http://localhost:8080/<file>

## Going live

Already live by default. Confirm the webhook in Settings, ensure the n8n workflow is Active, and use Test connection.

## One reader per page

A USB RFID reader is owned by a single browser page at a time. On one laptop, only one app can hold the Arduino — close the others' tabs. In production each station has its own reader.

Apollo Solar Ventures Inc.  ·  NUTECH Hardware & Software Solutions